GRADD Setup

We have written a GRADD filter that allows us to take advantage of the Hauppauge

PCI TV card's ability to blit video directly into your VGA card's memory space.

This allows video to be displayed with very little CPU overhead.  We recommend

using this mode only after you have verified basic driver functionality with

DiveTV.

1) Minimum system requirements :

Warp 4 Fixpack 6 or greater

GRADD support installed

Video Controller Card installed with sufficient memory to hold an entire screen

of information in a single linear aperture (i.e. about 2MB of RAM)

We are currently testing with Warp 4 Fixpack 12

2) To install

Assuming you have already installed the wcast.sys driver and have verified it is

functioning properly all that is necessary is for you to modify your config.sys

file. Backup config.sys edit config.sys changing the line

set C1=gradd-driver

to

set C1=gradd-driver,vcafilt

3) reboot machine

Now you are ready to use the WinCast driver and the GRADD extension.  Note that

you will need an application that knows how to use the driver in this mode.

DiveTV.EXE will not function any differently once you install the GRADD filter.

Instead you will need EX4.EXE from our video toolkit(a separate download):

X:\Toolkit Install Dir\SAMPLES\EX4\EX4.EXE

or one of the third party applications that are available.  We recommend using

one of these applications instead of EX4.EXE.  We have tested two applications:

StWTV - http://home.t-online.de/home/stefan.milcke/stwtv_en.htm

YADTV/2 - http://www3.mb.sympatico.ca/~lsunley/

Warning:  The GRADD filter is still under development.  Because it gives the

WinCast driver direct access to your video card memory you may see "screen junk"

drawn all over your desktop.  There have been reports of system lock up as well.

We will be addressing these issues as soon as possible.

Some video controllers work better than others. If when you go to a full screen

OS/2 session and it goes blank - type :

mode co80

If you are having difficulty setting up your Hauppauge TV card with these

drivers you can contact us by sending mail to:

os2tv-support@os2tv.com

Please use an appropriate subject heading and append output from the two

supplied utilities :

vxpenum wcast 1 > debug.doc

diag wcast 1   >> debug.doc

In addition, tell us the Version of OS/2 you are using, the make and model of

your VGA card, VGA card drivers used and resolutions tested.